Specification for equipment and materials for farmer’s biogas plant with gas holder, automated preparation of substrate, pneumatic feed and agitation and with heating of substrate

(July 2011)

| № | Materials and equipment | Quantity per plant |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5m3 | 10m3 | 15m3 | 25m3 | 50m3 | 100m3 |
|  | Digester | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Water boiler | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Gas burner (infrared radiation) | 1 | 1 | 1 | 1 | 2 | 4 |
|  | Moisture separator | 1 | 1 | 1 | 1 | 2 | 4 |
|  | Storage tank: vessel 3-5m3, concrete, metal | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Mixing and loading tank: vessel 0,5-1m3, metal, pressure 8 kg/cm2 | - | - | - | 1 | 1 | 1 |
|  | Compressor IF-56 with driving gear for biogas plant of 5-25 м3; FU-12 for plants with 50-100 м3 digesters | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Receiver: from 0,05 to 0,5м3, pressure up to 25 kg/cm2 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Gas holder of volume of 3м3 or above, pressure 8-25 kg/cm2 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Vacuum pump | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Digester | - | - | - | 1 | 1 | 1 |
|  | Heating system | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Agitation system | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Level meter | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Mercurial glass thermometer 0-1000 С, | 1 | 2 | 2 | 2 | 2 | 4 |
|  | Temperature level | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Manometers: electro contact. 0 - 1 kg/cm2 | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Manometer d = 100mm from -1 up to +5 kg/sm2 | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Manometer d = 100mm from 0 up to 25 kg/sm2 | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Manometer d = 150mm from 0 up to 1 kg/sm2 | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Manometer d = 60mm from 0 up to 10 kg/sm2 | - | - | - | 1 | 1 | 2 |
|  | Manometer d = 100mm from 0 up to 15 kg/sm2 | 3 | 3 | 3 | 3 | 3 | 3 |
|  | Water valve: d = 25mm | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Gas valve d = 15mm | 10 | 10 | 10 | 10 | 12 | 15 |
|  | Gas valve d = 25mm | 5 | 6 | 7 | 8 | 4 | 4 |
|  | Gas valve d = 32mm | 1 | 1 | 1 | 1 | 6 | 7 |
|  | Gas valve d = 40,50mm | - | - | - | - | 1 | 1 |
|  | Steel pipes (m) d = 15mm | 10 | 25 | 30 | 30 | 50 | 100 |
|  | Steel pipes (m) d = 25mm | 40 | 45 | 50 | 60 | 20 | 30 |
|  | Steel pipes (m) d = 32mm | 4 | 5 | 6 | 7 | 60 | 80 |
|  | Steel pipes (m) d = 50mm | 5 | 8 | 10 | 10 | 10 | 20 |
|  | Steel pipes (m) d = 100 или 89 mm | 30 | 30 | 30 | 40 | 110 | 220 |
|  | Steel pipes (m) d = 150mm | - | - | - | 5 | 5 | 5 |
|  | Catch d = 100mm | 4 | 4 | 4 | 5 | 5 | 7 |
|  | Catch d = 150mm | - | - | - | 1 | 1 | 1 |
|  | Flanges d = 100mm | 8 | 8 | 8 | 10 | 12 | 16 |
|  | Flanges d = 150mm | - | - | - | 2 | 2 | 2 |
|  | Fixings (kg): bolts m10, m12, m16 | 5 | 7 | 8 | 10 | 15 | 20 |
|  | Fixings (kg): washers 10, 12, 16 | 1 | 1 | 1,5 | 2 | 3 | 4 |
|  | Packing materials (m2): rubber, paronite | 2 | 2 | 2,5 | 3 | 4 | 5 |
|  | Pipe-bends F-15 | 12 | 12 | 14 | 16 | 18 | 25 |
|  | Pipe-bends F -25 | 10 | 10 | 12 | 14 | 16 | 20 |
|  | Pipe-bends F -32 | 2 | 2 | 2 | 4 | 8 | 12 |
|  | Pipe-bends F -50 | 10 | 10 | 12 | 12 | 12 | 24 |
|  | Pipe-bends F -100 or 89 | 4 | 6 | 6 | 24 | 24 | 44 |
|  | Adapter F 100 | - | - | - | 3 | 3 | 4 |
|  | Gas reducer with manometer | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Hydrogen sulphide filter | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Electrical level meter | 2 | 2 | 2 | 2 | 2 | 3 |
|  | Steel sheet 10-12mm (m2) | 2 | 2 | 2 | 2 | 2 | 4 |
|  | Steel sheet 4mm (m2) | 4 | 4 | 6 | 7 | 8 | 16 |
|  | Electrodes: 3mm (Chinese), kg | 30 | 30 | 35 | 35 | 60 | 90 |
|  | Safeguarding equipment | 2 | 2 | 2 | 1 | 1 | 2 |
|  | Safeguarding valve | 4 | 4 | 4 | 4 | 4 | 4 |
|  | Electrical cabinet | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Automation 3-f 39А, 50 А | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Actuators | 2 | 5 | 5 | 5 | 6 | 6 |
|  | Thermo switch | 2 | 5 | 5 | 5 | 6 | 6 |
|  | Starting relay | 2 | 5 | 5 | 5 | 6 | 6 |
|  | Buttons: “stop” red | 2 | 2 | 2 | 2 | 2 | 2 |
|  | Button “start” black | 2 | 2 | 2 | 2 | 2 | 2 |
|  | Lowering transformer 380 / 24÷36 В | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Terminal block for 24÷30 contacts | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Electrical driving gear (m) | 30 | 40 | 45 | 50 | 70 | 90 |
|  | Oxygen, propane | 2 | 2 | 3 | 3 | 5 | 8 |

Cost estimate for construction of farmer’s biogas plant with gas holder, automated preparation, pneumatic feed and agitation of substrate and heating of substrate

(see Figures 32 and 33)

| Name | Thousand som |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5m3 | 10m3 | 15m3 | 25m3 | 50m3 | 100m3 |
| - Digester | 20 | 30 | 40 | 60 | 120 | 240 |
| - Gas holder | 20 | 30 | 40 | 60 | 115 | 230 |
| - Compressor | 15 | 15 | 19 | 19 | 27 | 54 |
| - Materials for isolating of digester, gas holder, feeding, agitation and heating equipment | 16 | 17 | 18 | 20 | 28 | 56 |
| - Storage tank | 16 | 18 | 20 | 22 | 22 | 36 |
| - Substrate feeding equipment | 12 | 14 | 14 | 14 | 25 | 25 |
| - Equipment for anaerobic agitation | 6 | 6 | 6 | 6 | 8 | 16 |
| - Heating equipment | 14 | 16 | 18 | 22 | 44 | 88 |
| - Water separator | 0,7 | 0,7 | 0,7 | 0,7 | 0,7 | 1,5 |
| - Hydrogen sulfide filter | 5 | 5 | 6 | 6 | 7 | 7 |
| - Reductor | 0,8 | 0,8 | 0,8 | 0,8 | 7 | 9 |
| - Control equipment for level, temperature and pressure | 6 | 6 | 7 | 7 | 8 | 10 |
| - Automatic gas collecting system | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 |
| - Safeguard equipment (level of storage hopper, pressure in the digester, gas holder) | 3 | 3 | 3 | 4,2 | 4,8 | 6 |
| - Control cabinet | 8 | 8 | 8 | 10 | 12 | 12 |
| - Additional materials | 8 | 8 | 9 | 9 | 12 | 20 |
| - Work | 85 | 95 | 105 | 150 | 207 | 278 |
| Total*: | 239 | 276 | 318 | 414,2 | 651 | 1092 |
| Total* in USD (45 Som per USD): | 5350 | 6200 | 7100 | 9200 | 14500 | 24300 |

* This estimation does not include transportation costs, construction works costs and taxes.